Packet Tracer - Designing and Implementing a VLSM Addressing Scheme

Topology

You will receive one of three possible topologies.

- Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Remote-Site1 | G0/0 | 192.168.72.129 | 255.255.255.240 | N/A |
|  | G0/1 | 192.168.72.97 | 255.255.255.224 | N/A |
|  | S0/0/0 | 192.168.72.145 | 255.255.255.252 | N/A |
| Remote-Site2 | G0/0 | 192.168.72.65 | 255.255.255.192 | N/A |
|  | G0/1 | 192.168.72.1 | 255.255.255.224 | N/A |
|  | S0/0/0 | 192.168.72.146 | 255.255.255.252 | N/A |
| Sw1 | VLAN 1 | 192.168.72.130 | 255.255.255.240 | 192.168.72.129 |
| Sw2 | VLAN 1 | 192.168.72.98 | 255.255.255.224 | 192.168.72.97 |
| Sw3 | VLAN 1 | 192.168.72.66 | 255.255.255.224 | 192.168.72.65 |
| Sw4 | VLAN 1 | 192.168.72.2 | 255.255.255.192 | 192.168.72.1 |
| User-1 | NIC | 192.168.72.142 | 255.255.255.240 | 192.168.72.129 |
| User-2 | NIC | 192.168.72.126 | 255.255.255.224 | 192.168.72.97 |
| User-3 | NIC | 192.168.72.94 | 255.255.255.224 | 192.168.72.65 |
| User-4 | NIC | 192.168.72.62 | 255.255.255.192 | 192.168.72.1 |

Objectives

Part 1: Examine the Network Requirements

Part 2: Design the VLSM Addressing Scheme

Part 3: Assign IP Addresses to Devices and Verify Connectivity

Background

In this activity, you are given a /24 network address to use to design a VLSM addressing scheme. Based on a set of requirements, you will assign subnets and addressing, configure devices and verify connectivity.

Examine the Network Requirements

Determine the number of subnets needed.

You will subnet the network address 192.168.72.0

The network has the following requirements:

User-1 LAN will require 7 host IP addresses

User-2 LAN will require 15 host IP addresses

User-3 LAN will require 29 host IP addresses

User-4 LAN will require 58 host IP addresses

How many subnets are needed in the network topology? 5

Determine the subnet mask information for each subnet.

Which subnet mask will accommodate the number of IP addresses required for User-1 Lan ? /28 subnet

How many usable host addresses will this subnet support? 14

Which subnet mask will accommodate the number of IP addresses required for User-2 Lan? /27 subnet

How many usable host addresses will this subnet support? 30

Which subnet mask will accommodate the number of IP addresses required for User-3 Lan ? /27 subnet

How many usable host addresses will this subnet support? 30

Which subnet mask will accommodate the number of IP addresses required for User-4 Lan ? /26 subnet

How many usable host addresses will this subnet support? 62

Which subnet mask will accommodate the number of IP addresses required for the connection between Remote-Site 1 and Remote-Site 2 ? /30 subnet 255.255.255.252

Design the VLSM Addressing Scheme

Divide the Lan. network based on the number of hosts per subnet.

Use the first subnet to accommodate the largest LAN. Lan 0 = User-4 Lan

Use the second subnet to accommodate the second largest LAN.  Lan 1 = User-3 Lan

Use the third subnet to accommodate the third largest LAN.  Lan 2 = User-2 Lan

Use the fourth subnet to accommodate the fourth largest LAN.  Lan 3 = User-1 Lan

Use the fifth subnet to accommodate the connection between Remote-Site1 and Remote-Site2 : Lan 4

Document the VLSM subnets.

Complete the Subnet Table, listing the subnet descriptions (e.g. Lan 0 - LAN), number of hosts needed, then network address for the subnet, the first usable host address, and the broadcast address. Repeat until all addresses are listed.

Subnet Table

| Subnet Description | Number of Hosts Needed | Network Address/CIDR | First Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| Lan 0 | 48 | .0 | .1 | .63 |
| Lan 1 | 29 | .64 | .65 | .95 |
| Lan 2 | 15 | .96 | .97 | .127 |
| Lan 3 | 7 | .128 | .129 | .143 |
| Lan 4 | 2 | .144 | .145 | .147 |

  - Document the addressing scheme.

    - Assign the first usable IP addresses to Remote-Site1 for the two LAN links and the WAN link.

    - Assign the first usable IP addresses to Remote-Site2 for the two LANs links. Assign the last usable IP address for the WAN link.

    - Assign the second usable IP addresses to the switches.

    - Assign the last usable IP addresses to the hosts.

Assign IP Addresses to Devices and Verify Connectivity

Most of the IP addressing is already configured on this network. Implement the following steps to complete the addressing configuration.

  - Configure IP addressing on Remote-Site1 LAN interfaces.

  - Configure IP addressing on Sw3, including the default gateway.

  - Configure IP addressing on User-4, including the default gateway.

  - Verify connectivity.

You can only verify connectivity from User-1, User-2 and User-4 However, you should be able to ping every IP address listed in the Addressing Table.

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Examine the Network Requirements | Step 1 | 1 |  |
|  | Step 2 | 4 |  |
| Part 1 Total |  | 5 |  |
| Part 2: Design the VLSM Addressing Scheme |  |  |  |
| Complete Subnet Table |  | 25 |  |
| Document Addressing |  | 40 |  |
| Part 2 Total |  | 65 |  |
| Packet Tracer Score |  | 30 |  |
| Total Score |  | 100 |  |

ID: __________________________________________________________________________